UNIT - V

Cellular Wireless Networks:

->Cellular radio is a technique that was developed to increase the capacity available for mobile radio telephone service.

->Prior to cellular radio, mobile radio telephone service was only provided by a high-power transmitter/receiver.

->A typical system would support about 25 channels with an effective radius of about 80 km.

->way to increase the capacity of the system is to use lower-power systems with shorter radius and to use numerous transmitters/receivers.

Principles Of Cellular Network:

- Layout of cellular network

- Operation of cellular systems

- Effects of mobile radio broadcast

- Fading in mobile environment

- Layout of cellular network:

-> essence of a cellular network is the use of multiple low-power transmitters, on the order of 100 W or less.

->Because the range of transmitter is small, an area can be divided into cells, each having its own antenna.

->Each cell is allocated a band of frequencies and is served by a base station, consisting of transmitter, receiver, and control unit.

->Adjacent cells are assigned different frequencies to avoid interference or crosstalk or cells are sufficiently apart from each other so as to use same band.

Shape of cells:

i) Square:

->Simplest layout of cellular network is square shape cells

->If the width of a square cell is d, then a cell has four neighbors at a distance d and four neighbors at a distance squareroot(2d) .

->It is useful when all adjacent antennas are at equidistance.

ii)Hexagon:

->A hexagonal pattern provides for equidistant antennas.

->radius of a hexagon is defined to be the radius of the circle that circumscribes it (equivalently, the distance from the center to each vertex; also equal to the length of a side of a hexagon.

->For a cell radius R, the distance between the cell center and each adjacent cell center is d = squareroot(3) R.

Frequency reuse:

->In a cellular system, each cell has a base transceiver. The trans-mission power is carefully controlled  to allow communication within the cell using a given frequency while limiting the power at that frequency that escapes the cell into adjacent ones.

->objective is to use the same frequency in other nearby cells, thus allowing the frequency to be used for multiple simultaneous conversations.

->10 to 50 frequencies are assigned to each cell, depending on the traffic.

Increasing capacity:

->more customers use the system, traffic may build up so that there are not enough frequencies assigned to a cell to handle its calls.

->number of approaches used are:

i) Adding new channels:

->when a system is set up in a region, not all of the channels are used.

->growth and expansion can be managed in an orderly fashion by adding new channels.

ii) Frequency borrowing:

->frequencies are taken from adjacent cells by congested cells.

iii) Cell splitting:

->According to technique, split each cell into smaller cells to increase the capacity of cell.

->Size of original cell is in range of 6.5-13km.

->in general limit of each smaller cell is 1.5 km.

Fig: Cell Splitting

iv) Cell sectoring:

->cell is divided into a number of wedge-shaped sectors, each with its own set of channels, typically three or six sectors per cell.

->Each sector is assigned a separate subset of the cell’s channels, and directional antennas at the base station are used to focus on each sector.

v) Microcells:

->As cells become smaller, antennas move from the tops of tall buildings to the tops of small buildings and even to lamp posts to form microcells.

->Due to this power will be reduced and it is good for city streets,along roads and it is also useful in our inside buildings.

- Operation of cellular systems:

Fig: Overview of Cellular System

->Each cell is served by base station (BS).

->BS includes an antenna, a controller, and a number of transceivers, for communicating on the channels assigned to that cell.

->Controller is used to handle the call process between the mobile unit and the rest of the network.

->Every BS is connected to a mobile telecommunications switching office (MTSO).

->One MTSO serves many BSs.

->Link between an MTSO and a BS is by a wire line, although a wireless link is also possible.

->MTSO connects calls between mobile units.

->MTSO can also be connected to the public telephone or telecommunications network and can make a connection between a fixed subscriber to the public network and a mobile subscriber to the cellular network.

->MTSO assigns the voice channel to each call, performs handoffs, and monitors the call for billing information.

->Use of a cellular system is fully automated and requires no action on the part of the user other than placing or answering a call.

->Two types of channels are available between the mobile unit and the base station (BS):

i) Control channels:

->Used to exchange information having to do with setting up and maintaining calls.

->Establish relationship between a mobile unit and the nearest BS.

ii) Traffic channels:

->Carry a voice or data connection between users.

Various steps in a typical call between two mobile users within an area controlled by a single MTSO:

i) Mobile unit initialization:

-> It examines and choose the strongest setup control channel.

-> It automatically selected the BS antenna of the cell.

->The handshake mechanism is used in between the mobile device and MTSO.

Fig: monitor for strongest signal

ii) Mobile-originated call:

->A mobile unit originates a call by sending the number of the called unit on the preselected setup channel.

->Receiver at the mobile unit first checks that the setup channel is idle by examining information in the forward (from the BS) channel.

->If channel is idle then mobile device sends request to BS and BS passes this request to MTSO.

Fig: Request for connection

iii) Paging:

->MTSO then attempts to complete the connection to the called unit.

->MTSO sends a paging message to certain BSs depending on the called mobile number.

->Each BS transmits the paging signal on its own assigned setup channel.

Fig: Paging

iv) Call accepted:

->Called device identifies its number from setup channel and gives response to mobile device through base station and MTSO.

->A circuit is created between the calling and called BSs by MTSO.

->MTSO choose a traffic channel within range of each base station cell and identifies all base stations.

->Hence connection is established between 2 mobile devices.

Fig: Call accepted

v) Ongoing call:

-> While the connection is maintained, the two mobile units exchange voice or data signals, going through their respective BSs and the MTSO.

Fig: Ongoing call

vi) Handoff:

->When a mobile device is out of covering area of particular cell and enters into the range of another cell while establishing a connection, then traffic channel has to transform to new cell without any interruption for call.

Fig: Handoff

Call blocking:

->During the mobile-initiated call stage, if all the traffic channels assigned to the nearest BS are busy, then the mobile unit makes a preconfigured number of repeated attempts.

->After a certain number of failed tries, a busy tone is returned to the user.

Call termination:

->When one of the two users hangs up, the MTSO is informed and the traffic channels at the two BSs are released.

Call drop:

->During a connection, because of interference or weak signal spots in certain areas, if the BS cannot maintain the minimum required signal strength for a certain period of time, the traffic channel to the user is dropped and the MTSO is informed.

Calls to/from fixed and remote mobile subscriber:

->MTSO connects to the public switched telephone network.

->Thus MTSO can make connection with a mobile user in its area and a fixed subscriber via the telephone net-work.

- Mobile radio propagation effects:

i) Strength of signal:

->The base station and mobile device should have to maintain the strength of signal in order to provide quality of signal at receiver.

->If quality of signal is so strong then it leads to co-channel interference with other cells.

->Strength of signal depends on mobile device,as we know mobile devices moves from one place to another.

ii) Signal broadcast effect (Fading):

->Sometimes it may happen that strength of signal is within a proper coverage area, signal broadcast effects may interrupt the signal and cause errors.

->This problem is called fading.

Types of fading:

->Fading effects in a mobile environment can be classified as either fast or slow.

->As mobile unit moves down a street in an urban environment rapid variations in signal strength occur over distance of about one half a wavelength.

->At frequency of 900MHz,which is typical for mobile cellular applications, wavelength is 0.33m.

- Fast fading:

->Changes of amplitude can be as much as 20 o 30 dB over a short distances.

->This type of rapidly changing fading phenomenon, known as fast fading.

b) Slow fading:

->It refers to slower changes due to mobile user who moves to different height buildings and gaps in between the buildings.

c) Nonselective (flat) fading:

->In this all frequency components of the received signal fluctuate in the same proportions simultaneously.

Effect of multipath propagation:

- Multiple copies of a signal may arrive at different phases.

- Particular for digital transmission, intersymbol interference (ISI).

- Sending a narrow pulse at a given frequency across a link between a fixed antenna and a mobile device, channel may deliver multiple copies at diff times.

Delayed pulses acts as noise,making recovery of bit information difficult.

d) As mobile device moves from one place to another then amount of LOS(line of sight) increases.

e) Very difficult to design signal processing to filter out multipath effects.

Since mobile antenna moves,location of various obstacles changes; hence the number,magnitude and timing of secondary pulses charge.

This make it difficult to design signal processing techniques.

Whenever an electromagnetic wave encounters that kind of an edge,then wave broadcast in different directions by making edge as source.

Scattering:

->It happens when there is delay in progress of wavelength of signal.

->Incoming signal is scattered into several weaker signals.

->scattering can also occur when lamp posts and traffic signs are used.

->If size of obstacle is on the order of wavelength of signal or less,scattering occurs.

Fig: Sketch of Three Important Propagation Mechanisms: Reflection (R), Scatter-ing (S),                            Diffraction (D)

- Fading in mobile environment:

->Fading refers to the time variation of received signal power caused by changes in the transmission medium or paths.

->In a fixed environment, fading is affected by changes in atmospheric conditions, such as rainfall.

->But in a mobile environment, where one of the two antennas is moving relative to the other, the relative location of various obstacles changes over time, creating complex transmission effects.

Multipath propagation:

->Consists of 3 important mechanism:

i) Reflection:

->It is electromagnetic signal which happens on surface that is corresponding to wavelength of signals.

ii) Diffraction:

->It occurs at surface of an object which is relatively large compared to wavelength of electromagnetic wave.

iii) Selective fading:

->It affects unequally the different spectral components of a radio signal.

-> If attenuation occurs over a portion of the bandwidth of the signal, the fading is considered to be selective.

First generation analog

->Original cellular telephone networks provided analog traffic channels, these are now referred to as first generation system.

->Most common first generation system in north America has been Advanced mobile phone service(AMPS) developed by AT&T.

->AMPS operates in 300MHz band.

System uses 2 separate analog channels, one for forward(BS to mobile station) communication and one for reverse(mobile station to BS) communication.

->Band width - 869-894MHz - forward communication

824-849MHz - reverse communication

->Each band is divided into 832 channels 21 out of 832 channels are used for control.

->AMPS has frequency reuse factor of 7, this means only one-seventh of these 395 traffic channels are actually available in a cell.

Transmission:

-> AMPS uses FM and FSK for modulation.

->Voice channels are modulated using FM and control channels use FSK to create 30KHz analog signals for this AMPS uses FDMA to divide each 25MHz band into 30KHz channels.

Operation:

->Each AMPS-capable cellular telephone includes a numeric assignment module (NAM) in read-only memory.

->NAM contains the telephone number of the phone, which is assigned by the service provider, and the serial number of the phone, which is assigned by the manufacturer.

->When the phone is turned on, it transmits its serial number and phone number to the MTSO.

->MTSO maintains a database with information about mobile units.

->MTSO uses the phone number for billing purposes.

Steps of events when call is placed:

i) Subscriber initiates a call by keying in the telephone number of the called party and presses the send key.

ii) MTSO verifies that the telephone number is valid and that the user is authorized to place the call; some service providers require the user to enter a PIN (personal identification number) as well as the called number to counter theft.

iii) MTSO issues a message to the user’s cell phone indicating which traffic channels to use.

iv) MTSO sends out a ringing signal to the called party.

All of these operations occur within 10 s of initiating the call.

v) When the called party answers, the MTSO establishes a circuit between the two parties and initiates billing information.

vi) When one party hangs up, the MTSO releases the circuit, frees the radio channels, and completes the billing information.

Second generation CDMA (2G):

->Second generation systems are digital.

->Second generation systems provide higher quality signals, higher data rates for support of digital services, and greater capacity.

->Some differences between the two generations:

i) Digital traffic channels:

->The most notable difference between the two generations is that first-generation systems are almost purely analog, whereas second-generation systems are digital.

->First generation systems are designed to support voice channels using FM whereas Second generation systems provide digital traffic channels which readily support digital data.

ii) Encryption:

->User and control traffic is digitized in second generation systems, so it is simple matter to encrypt all traffic to prevent eavesdropping whereas first-generation systems send user traffic the clear, providing no security.

iii) Error detection and correction:

->2G uses error detection and correction techniques.

iv) Channel access:

->2G uses time division multiple access (TDMA) or code division multiple access (CDMA).

Code division multiple access:

->In FDMA each cell is allocated a frequency bandwidth, which is split into two parts, half for reverse (mobile unit to base station) and half for forward (base station to mobile unit).

->For full-duplex communication, a mobile unit uses both reverse and forward channels.

->CDMA for cellular systems uses direct-sequence spread spectrum (DS-SS), which uses a chipping code to increase the data rate of the transmission, resulting in an increased signal bandwidth.

->Multiple access is provided by assigning orthogonal chipping codes to multiple users, so that the receiver can recover the transmission of an individual unit from multiple transmissions.

Advantages of CDMA:

i) Frequency diversity:

-> Because the transmission is spread out over a larger bandwidth, frequency-dependent transmission impairments, such as noise bursts and selective fading, have less effect on the signal.

ii) Multipath resistance:

->Chipping codes used for CDMA not only exhibit low cross correlation but also low autocorrelation.

Therefore, a version of the signal that is delayed by more than one chip interval does not interfere with the dominant signal as much as in other multipath environments.

iii) Privacy:

->Each user has a unique code, privacy is inherent.

iv) Graceful degradation:

->With CDMA, as more users access the system simultaneously, the noise level and hence the error rate increases; only gradually does the system degrade to the point of an unacceptable error rate.

Drawbacks of CDMA:

i) Self-jamming:

->Unless all of the mobile users are perfectly synchronized, the arriving transmissions from multiple users will not be perfectly aligned on chip boundaries.

->Thus the spreading sequences of the different users are not orthogonal and there is some level of cross correlation.

->In TDMA or FDMA, there is reasonable gap (guard bands of frequency or time) therefore received signals are orthogonal or nearly so.

ii) Near-far problem:

-> Signals closer to the receiver are received with less attenuation than signals farther away.

Third generation system (3G) :

->The objective of the third generation (3G) of wireless communication is to provide fairly high-speed wireless communications to support multimedia, data, and video in addition to voice.

3G capabilities:

i) Voice quality comparable to the public switched telephone network.

ii) Support for 2.048 Mbps for office use

iii) Symmetrical and asymmetrical data transmission rates.

iv) Support for both packet-switched and circuit-switched data services.

v) More efficient use of the available spectrum in general .

vi) Support for a wide variety of mobile equipment.

vii) Flexibility to allow the introduction of new services and technologies.

->Alternative schemes that have been adopted as part of IMT 2000 which covers a set of radio interfaces for optimized performance in different radio environments.

i) W-CDMA or wideband CDMA:

fully exploits CDMA technology to provide high data rates with efficient use of bandwidth.

ii) CDMA 2000

iii) TD-CDMA

iv) TDD (TDMA based network)

v) DECT (FDMA based network)

Bluetooth:

->Bluetooth is a wireless LAN technology designed to connect devices of different functions

such as telephones, notebooks, computers (desktop and laptop), cameras, printers,

coffee makers, and so on.

->Bluetooth LAN is an ad hoc network, which means that the

network is formed spontaneously; the devices, sometimes called gadgets, find each

other and make a network called a piconet.

->Bluetooth LAN network cannot be large. If there are many gadgets that try to connect, there is chaos.

Applications:

i) Peripheral devices such as a wireless

mouse or keyboard can communicate with the computer through this technology.

ii) Monitoring devices can communicate with sensor devices in a small health care center.

iii) Home security devices can use this technology to connect different sensors to the main security controller.

iv) Conference attendees can synchronize their laptop computers at a conference.

->Bluetooth technology is the implementation of a protocol defined by the IEEE 802.15 standard which defines a wireless personal-area network (PAN) operable in an area the size of a room or a hall.

Architecture:

-> Bluetooth defines two types of networks:

i) piconet

ii)scatternet

i) Piconet:

-> A piconet can have up to eight stations, one of which is called the primary and the rest are called secondaries.

-> All the secondary stations synchronize their clocks with primary.

-> piconet can have only one primary station.

-> communication between the primary and the secondary can be one-to-one or one-to-many.

-> Maximum number of secondaries=7   +  1 primary i.e total =8

-> Additional 8 secondaries can be added in parked state.

-> A secondary in a parked state is synchronized with the primary, but cannot take part in communication until it is moved from the parked state.(only eight stations can be active in a piconet)

ii) Scatternet:

-> Piconets can be combined to form a scatternet.

-> Secondary station in one piconet can be the primary in another piconet.

-> This station can receive messages from the primary in the first piconet (as a secondary) and, acting as a primary, deliver them to secondaries in the second piconet.

Bluetooth Devices:

-> Bluetooth device has a built-in short-range radio transmitter.

-> current data rate is 1 Mbps with a 2.4-GHz bandwidth.

-> There is a possibility of interference between the IEEE 802.11b wireless LANs and Bluetooth LANs.

Bluetooth Layers:

-> Bluetooth uses several layers that do not exactly match those of the Internet model.

i) Radio Layer:

-> It is equivalent to the physical layer of the Internet model.

-> Bluetooth devices are low-power and have a range of 10 m.

-> Bluetooth uses a 2.4-GHz ISM band divided into 79 channels of 1 MHz each.

-> Bluetooth uses the frequency-hopping spread spectrum (FHSS) method in the physical

layer to avoid interference from other devices or other networks.

-> To transform bits to a signal, Bluetooth uses a sophisticated version of FSK, called

GFSK (FSK with Gaussian bandwidth filtering.

ii) Baseband layer:

-> It is equivalent to the MAC sublayer in LANs.

-> access method is TDMA, The primary and secondary communicate with

each other using time slots.

-> communication is only between the primary and a secondary; secondaries cannot communicate directly with one another.

-> TDMA: Bluetooth uses a form of TDMA that is called TDD-TDMA (timedivision duplex TDMA).

-> TDD-TDMA is a kind of half-duplex communication in which the secondary and receiver send and receive data, but not at the same time (halfduplex).

-> communication for each direction uses different hops:

- Single secondary communication - primary uses even slots

- secondary uses odd slots

(b) Multiple secondary communication

Dwell time - 625 microseconds

-> Bluetooth hops 1600 times/second , therefore each device changes its modulation frequency 1600 times/second.

-> Device uses frequency for only 625 microseconds(1/1600 sec) before it hops to another frequency.

Physical links:

->Two types of links can be created between a primary and a secondary:

- SCO links (synchronous connection-oriented)

- ACL links (asynchronous connectionless link)

-> SCO used when avoiding delay in data delivery is more important than error free delivery(integrity).

-> ACL used when integrity is more important.

Frame format:

-> A frame in baseband layer can be one of three types:

(i) One-slot

(ii) Three-slot

(iii) Five-slot

-A slot is 625 microseconds

iii) L2CAP:

-> Logical Link Control and Adaptation Protocol, or L2CAP (L2 here means LL).

-> equivalent to the LLC sublayer in LANs.

-> It is used for data exchange on an ACL link.

-> L2CAP data packet format :

-> L2CAP has specific duties:

- Multiplexing

- Segmentation

- Reassembly

- QOS(quality of service)

- Group management

WIRELESS LANs:

-> Wireless LANs makes use of wireless transmission medium.

-> Wireless LANs were less used until relatively because of high prices,low data rates,occupational safety concerns and licensing requirement.

-> As these problems have been addressed popularity of wireless LANs has grown rapidly.

Applications:

- LAN extension

- Cross-Building Interconnect

- Nomadic Access

- Ad Hoc Networking

- LAN extension:

-> Wireless LANs saves cost of cabling and eases task of relocation and other modifications to network structures.

-> However, this motivation for wireless LANs was overtaken by events.

(i)as awareness of the need for LANs became greater,architects designed new buildings to include extensive prewiring for data applications.

(ii)with advances in data transmission technology, there is an increasing reliance on twisted pair cabling for LANs and, in particular, Category 3 and Category 5 unshielded twisted pair.

(iii)Thus, the use of a wireless LAN to replace wired LANs has not happened to any

great extent.

(b) Cross-Building Interconnect:

-> connect LANs in nearby buildings, be they wired or wireless LANs.

-> point-to-point wireless link is used between two buildings.

(c) Nomadic Access:

-> Nomadic access provides a wireless link between a LAN hub and a mobile data terminal equipped with an antenna, such as a laptop computer or notepad computer.

(d) Ad Hoc Networking:

-> An ad hoc network is a peer-to-peer network (no centralized server) set up temporarily to meet some immediate need.

-> For example, a group of employees, each with a laptop or palmtop computer, may convene in a conference room for a business or classroom meeting.

-> The employees link their computers in a temporary network just for the duration of the meeting.

-> In number of environments,there is a role for wireless LAN as alternative to wired LAN.

Example:

(i) Building with large open areas such as manufacturing plants,stock exchange trading floors and warehouses.

(ii) Historical buildings with insufficient twisted pair of wire and where drilling for new wiring is prohibited.

(iii) Small offices where installation and maintenance of wired LANs is not economical.

Requirement of  wireless LAN:

- Throughput: medium access control (MAC) protocol should make as efficient use as possible of the wireless medium to maximize capacity.

- Number Of Nodes: support hundreds of nodes across multiple cells.

- Connection to backbone LAN: For infrastructure wireless LANs, this is easily accomplished through the use of control modules that connect to both types of LANs.

- Service Area: A typical coverage area for a wireless LAN has a diameter of 100 to 300 m.

- Battery Power Consumption: Mobile workers use battery-powered workstations that need to have a long battery life when used with wireless adapters.

Typical wireless LAN implementations have features to reduce power consumption while not using the network, such as a sleep mode.

- Transmission robustness and security: design of a wireless LAN must permit reliable transmission even in a noisy environment and should provide some level of security from eavesdropping.

7) Collocated network operation: when two or more wireless LANs to operate in the same area or in some area where interference between the LANs is possible. Such interference may disturb the normal operation of a MAC algorithm and may allow unauthorized access to a particular LAN.

8) License-free operation: Users would prefer to buy and operate wireless LAN products without having to secure a license for the frequency band used by the LAN.

9) Handoff/roaming: The MAC protocol used in the wireless LAN should enable mobile stations to move from one cell to another.

10) Dynamic configuration: The MAC addressing and network management aspects of the LAN should permit dynamic and automated addition, deletion, and relocation of end systems without disruption to other users.

Wireless LAN technology:

-> Wireless LANs are generally categorized according to the transmission technique that is used.

(i) Infrared LANs (IR LANs):

-> An individual cell of an IR LAN is limited to a single room, because infrared light does not penetrate opaque walls.

-> Optical wireless communication in the infrared portion of the spectrum is commonplace in most homes, where it is used for a variety of remote control devices.

Strengths and weakness:

-> The two competing transmission media for wireless LANs are microwave radio using either spread spectrum or narrow band transmission,infrared.

- spectrum for infrared is virtually unlimited, which presents the possibility of achieving extremely high data rates. The infrared spectrum is unregulated worldwide.

(b) infrared shares some properties of visible light that make it attractive for certain types of LAN configurations.

-> Infrared light is diffusely reflected by light-colored objects; thus it is possible to use ceiling reflection to achieve coverage of an entire room.

-> Infrared light does not penetrate walls or other opaque objects.

This has two advantages:

- infrared communications can be more easily secured against eavesdropping than microwave.

- separate infrared installation can be operated in every room in a building without interference, enabling the construction of very large infrared LANs.

-> infrared equipment is relatively inexpensive and simple.

-> Infrared data transmission typically uses intensity modulation, so that IR receivers need to detect only the amplitude of optical signals, whereas most microwave receivers must detect frequency or phase.

Transmission Techniques:

- Directed-beam IR:

->used to create point-to-point links.

->range depends on the emitted power and on the degree of focusing.( remote TV control)

- Omnidirectional configuration:

-> involves a single base station that is within ine of sight of all other stations on the LAN.

-> this station is mounted on the ceiling.

- Diffused configuration:

-> all of the IR transmitters are focused and aimed at a point on a diffusely reflecting ceiling. IR radiation striking the ceiling is reradiated omnidirectionally and picked up by all of the receivers in the area.

(ii) Spread Spectrum LANs:

-> This type of LAN makes use of spread spectrum transmission technology.

-> Except for quite small offices, a spread spectrum wireless LAN makes use of a multiple-cell arrangement.

-> Adjacent cells make use of different center frequencies within the same band to avoid interference.

-> Within a given cell, the topology can be either hub or peer to peer.

-> Regardless of access control mechanism,each station may broadcast using an omnidirectional antenna so that all other stations in cell may receive.

-> A peer-to-peer topology is one in which there is no hub. CSMA is used to control access.

-> appropriate for ad hoc LANs.

-> Wireless LAN can be usable without having to go through a licensing procedure.

(iii) Narrowband Microwave LANs:

-> Narrowband microwave refers to the use of microwave radio frequency band for signal transmission,with relatively narrow bandwidth

-> just wide to accomodate signal.

-> Licensed Narrowband RF

Unlicensed Narrowband RF

IEEE 802.11

-> IEEE has defined specifications for wireless LAN,called IEEE 802.11,which covers physical and data link layers.

Architecture:

-> Std. Define 2 services:

(i) Basic service set (BSS)

(ii) Extended service set (ESS)

(i) BSS:

-> It is made of stationary or mobile wireless stations and optional central base station called Access point (AP).

-> BSS without AP,is stored-alone network and cannot send data to other BSSs.

-> It is called an ad hoc architecture.

-> BSS with an AP is an infrastructure network.

(ii) ESS:

-> It is made up of two or more BSSs with APs.

-> In this case, the BSSs are connected through a distribution system, which is usually a wired

LAN.

-> distribution system connects the APs in the BSSs.

-> distribution system can be any IEEE LAN such as an Ethernet.

-> ESS uses 2 types of stations:

- mobile (normal station inside BSS)

- stationary (AP stations that are part of wired LAN)

-> Communication between 2 stations in 2 different BSSs usually occurs via 2 APs.

Station Types:

-> three types of stations according to IEEE 802.11 based on their mobility in a wireless LAN:

- No Transition:

-> Station with no transition mobility is either stationary or moving inside BSS.

(b) BSS Transition:

-> Move from 1 BSS to another BSS inside 1 BSS.

(c) ESS transition mobility:

-> Move from 1 BSS to another.

MAC Sublayer:

-> IEEE 802.11 defines two MAC sublayers:

(a)the distributed coordination function (DCF)

(b)point coordination function (PCF).

- DCF:

-> DCF uses CSMAICA as the access method. Wireless LANs cannot implement CSMAfCD for three reasons:

(i) For collision detection a station must be able to send data and receive collision signals at the same time.

This can mean costly stations and increased bandwidth requirements.

(ii) Collision may not be detected because of the hidden station problem.

(iii) distance between stations can be great.

Signal fading could prevent a station at one end from hearing a collision at the other end.

Process Flowchart:

-> Fig shows flowchart for CSMA/CA as used in wireless LANs

DIFS- distributed interframe space

SIFS- Short interframe space

RTS- request to send

CTS- clear to send

(b) PCF:

-> PCF has a centralized, contention-free polling access method.

-> AP performs polling for stations that are capable of being polled.

-> stations are polled one after another, sending any data they have to the AP.

-> To give priority to PCF over DCF, another set of interframe spaces has been defined:

(i)PIFS-shorter than DIFS

(ii) SIFS-same as that in DCF

-> This means that if, at the same time, a station wants to use only DCF and an AP wants to use PCF, the AP has priority.

-> Due to the priority of PCF over DCF, stations that only use DCF may not gain access to the medium.

-> To prevent this, a repetition interval has been designed to cover both contention-free (PCF) and contention-based (DCF) traffic.

-> repetition interval,which is repeated continuously, starts with a special control frame, called a beacon frame.

-> When the stations hear the beacon frame, they start their NAV for the duration of the contention-free period of the repetition interval.

-> During the repetition interval, the PC (point controller) can send a poll frame, receive data, send an ACK, receive an ACK, or do any combination of these.

-> At the end of the contention-free period, the PC sends a CF end (contention-free end) frame to allow the contention-based stations to use the medium.

-> Fragmentation:

division of a large frame into smaller ones. It is more efficient to resend a small frame than a large one.

Frame Format:

-> MAC frame consists of 9 fields

(i) FC- type of frame and some control information(frame control)

Subfields:

(ii) D- In all frame types except one, this field defines the duration of the transmission that is used to set the value of NAY.

In one control frame, this field defines the ID of the frame.

(iii) Addresses- four address fields, each 6 bytes long.

The meaning of each address field depends on the value of the To DS and From DS subfields

(iv) Sequence control- This field defines the sequence number of the frame to be used in flow control.

(v) Frame body- between 0 and 2312 bytes, contains information based on the type and the subtype defined in the FC field.

(vi)FCS- CRC-32 error detection sequence.

Frame Types:

-> IEEE 802.11 has three categories of frames types for wireless LAN

(i) Management frame- used for initial communication between station and APs

(ii) Control frame- used for accessing channels and acknowledging frames

(iii) Data frame- carrying data and control information

Control Frames:

-> For control frame value of type field is 01.

-> values of subtype fields for frames

Addressing mechanism:

-> The IEEE 802.11 addressing mechanism specifies four cases, defined by the value of the two flags in the FC field, To DS and From DS.

-> Each flag can be either 0 or I, resulting in four cases.

-> Add 1-always the address of next device

Add 2-always address of previous device

Add 3-address of final destination,if it is not specified by add 1

Add 4-address of original source,if it s not same as add 2

Case 1: 00

->In this case, To DS = 0 and From DS = O. This means that the frame is not going to a distribution system (To DS = 0) and is not coming from a distribution system(from DS=0)

-> frame is going from one station in a BSS to another without passing through the distribution system.

-> ACK frame should be sent to the original sender.

Case 2: 01

-> To DS = 0 and From DS = 1

-> frame is coming from a distribution system (From DS = 1)

->frame is coming from an AP and going to a station.

->ACK should be sent to the AP.

Case 3: 10

->To DS =1 and From DS =O.

->frame is going to a distribution system (To DS = 1).

->frame is going from a station to an AP.

-> The ACK is sent to the original station.

Case 4: 11

-> To DS =1 and From DS =1.

-> This is the case in which the distribution system is also wireless.

-> The frame is going from one AP to another AP in a wireless distribution system.

-> We do not need to define addresses if the distribution system is

a wired LAN because the frame in these cases has the format of a wired LAN frame

(Ethernet, for example).

Physical layer:

->Six Specifications:

-> All implementations, except the infrared, operate in the industrial, scientific,

and medical (ISM) band, which defines three unlicensed bands in the three ranges

902-928 MHz, 2.400--4.835 GHz, and 5.725-5.850 GHz.